Performance & Load Testing Report

Apache JMeter — CSE214 Offline Assignment 4

Student ID: 2305025  |  Submission Date: 18 September, 2026

📹  Video Demonstration

YouTube Link:  https://youtu.be/EVuboAgu5LM

The video demonstrates the full JMeter workflow — test plan configuration, HTTP(S) Script Recorder setup, execution of both 50-user and 100-user load profiles, observation of Duration Assertion pass/fail events, and generation of the HTML dashboard report. Webcam picture-in-picture is included throughout the recording.

1.  Introduction

This report presents a comprehensive quantitative performance evaluation of the LoadLab web application (http://103.94.135.91:8080) using Apache JMeter. Two load profiles were executed — 50 concurrent users and 100 concurrent users — each with a ramp-up period of 100 seconds and a Duration Assertion threshold of 200 ms applied to all five target endpoints. The following sections analyse execution time, throughput, bandwidth, and assertion/error behaviour across both profiles.

Target Endpoints

- GET  /  —  Home Page — baseline static page (~2 KB response)

- GET  /notices  —  Notice Board — dynamic content with random 100–400 ms latency (~7 KB)

- GET  /courses  —  Course Catalogue — CPU-intensive, load-scalable latency (~43 KB)

- POST /login  —  User Authentication — form POST with 303 redirect

- GET  /api/download/256  —  Bandwidth Endpoint — high-throughput download test (256-byte body)

2.  Execution Time Analysis

Table 1 provides a side-by-side comparison of the minimum, maximum, and average response times (in milliseconds) recorded for each of the five endpoints under both load profiles. All timing values are sourced directly from the JMeter HTML Dashboard statistics.

Table 1: Execution Time Comparison — 50 vs. 100 Users (milliseconds)

| Target Endpoint | Samples | Min (ms) | Max (ms) | Avg (ms) | Throughput (req/s) | Error % |
| --- | --- | --- | --- | --- | --- | --- |
| Home Page (GET /) | 50 | 21 | 1615 | 320.4 | 0.5098 | 24.0% |
|  | 100 | 22 | 1150 | 221.9 | 1.0099 | 23.0% |
| Notice Board (GET /notices) | 50 | 125 | 856 | 380.5 | 0.5120 | 82.0% |
|  | 100 | 171 | 1578 | 436.2 | 1.0097 | 88.0% |
| Course Catalogue (GET /courses) | 50 | 213 | 1278 | 443.9 | 0.5120 | 100.0% |
|  | 100 | 245 | 1897 | 519.2 | 1.0081 | 100.0% |
| User Login (POST /login) | 50 | 14 | 550 | 71.1 | 0.5160 | 6.0% |
|  | 100 | 7 | 739 | 97.2 | 1.0202 | 9.0% |
| Data Transfer (GET /api/download/256) | 50 | 54 | 2237 | 591.5 | 0.5126 | 72.0% |
|  | 100 | 94 | 3938 | 791.0 | 1.0068 | 80.0% |
| TOTAL (All Endpoints) | 50 | 14 | 2237 | 361.5 | 2.4990 | 56.8% |
|  | 100 | 7 | 3938 | 413.1 | 4.9398 | 60.0% |

Key Observations — Execution Time

- Home Page (GET /): Average response time went from 320.4 ms (50 users) to 221.9 ms (100 users). Despite doubling concurrency, the server handled the static page more efficiently — likely due to connection reuse and caching. The maximum dropped from 1615 ms to 1150 ms.

- Notice Board (GET /notices): Average latency increased from 380.5 ms (50 users) to 436.3 ms (100 users), as expected from a server-injected 100–400 ms random delay endpoint. The maximum also rose sharply from 856 ms to 1578 ms, reflecting queuing effects under higher concurrency.

- Course Catalogue (GET /courses): This CPU-intensive endpoint showed increased averages (443.9 ms → 519.2 ms) and a higher maximum (1278 ms → 1897 ms) under 100 users, confirming that CPU-bound latency scales with concurrency. It triggered a 100% assertion failure rate in both runs.

- User Login (POST /login): Average response time increased from 71.1 ms (50 users) to 97.2 ms (100 users), with the maximum rising from 550 ms to 739 ms. The error rate slightly increased (6% → 9%), reflecting session handling overhead under doubled load.

- Data Transfer (GET /api/download/256): Average response time worsened significantly from 591.5 ms (50 users) to 791.0 ms (100 users). The maximum nearly doubled from 2237 ms to 3938 ms, confirming that bandwidth-heavy requests are the most sensitive to concurrency increases.

3.  Throughput & Bandwidth Analysis

Table 2 presents the average throughput (requests/sec), data reception rate (KB/sec), and data transmission rate (KB/sec) for each endpoint and for the combined totals under both load profiles.

Table 2: Throughput & Bandwidth Comparison — 50 vs. 100 Users

| Target Endpoint | Samples | Throughput (req/s) | Recv Rate (KB/s) | Send Rate (KB/s) |
| --- | --- | --- | --- | --- |
| Home Page (GET /) | 50 | 0.5098 | 1.2118 | 0.1752 |
|  | 100 | 1.0099 | 2.4006 | 0.3837 |
| Notice Board (GET /notices) | 50 | 0.5120 | 3.7593 | 0.1980 |
|  | 100 | 1.0097 | 7.4133 | 0.3905 |
| Course Catalogue (GET /courses) | 50 | 0.5120 | 22.2015 | 0.2015 |
|  | 100 | 1.0081 | 43.7155 | 0.3967 |
| User Login (POST /login) | 50 | 0.5160 | 0.7785 | 0.2207 |
|  | 100 | 1.0202 | 1.5393 | 0.3995 |
| Data Transfer (GET /api/download/256) | 50 | 0.5126 | 131.3625 | 0.1842 |
|  | 100 | 1.0068 | 258.0089 | 0.3618 |
| TOTAL (All Endpoints) | 50 | 2.4990 | 155.3694 | 0.9552 |
|  | 100 | 4.9398 | 307.1224 | 1.8881 |

Key Observations — Throughput & Bandwidth

- Overall throughput nearly doubled from 2.4990 req/s (50 users) to 4.9398 req/s (100 users), scaling almost linearly with the doubling of concurrent threads. This confirms the server can sustain proportional throughput scaling at this concurrency level.

- The Data Transfer endpoint (GET /api/download/256) dominated bandwidth with a reception rate of 131.36 KB/s (50 users) that rose dramatically to 258.01 KB/s (100 users) — nearly doubling in line with the thread count increase. This constitutes ~84% of total received bandwidth in the 100-user run.

- Course Catalogue reception rate nearly doubled (22.20 KB/s → 43.72 KB/s), consistent with 100 users each downloading the ~43 KB HTML payload in approximately the same time window.

- Data transmission (sent) rates also scaled proportionally — total sent increased from 0.9552 KB/s (50 users) to 1.8881 KB/s (100 users) — roughly 2× as expected when doubling the number of concurrent request senders.

- Total data reception nearly doubled from 155.37 KB/s → 307.12 KB/s (+97.7%), demonstrating near-linear I/O scaling from 50 to 100 users. This suggests the server has not yet reached I/O saturation at 100 concurrent users.

4.  Assertion & Error Rate Analysis

4.1  Duration Assertion Configuration

A Duration Assertion was applied globally to all five HTTP request samplers with a maximum allowed duration threshold of 200 milliseconds. This threshold was deliberately set below the observed baseline averages for most endpoints (e.g., /notices avg ≈ 380 ms, /courses avg ≈ 444 ms, /api/download/256 avg ≈ 591 ms) to ensure that a mix of assertion passes and failures would be observed during test execution — as required by the assignment specification.

4.2  Error Rate Summary Table

Table 3: Assertion Failure / Error Rate Summary

| Endpoint | 50 Users Error % | 100 Users Error % | Δ Change | Likely Cause |
| --- | --- | --- | --- | --- |
| GET / | 24.0% | 23.0% | −1.0% | Static page; near-stable error rate — most responses still exceed 200 ms threshold at both loads |
| GET /notices | 82.0% | 88.0% | +6.0% | Server-injected random delay causes more queuing at 100 users; additional responses exceed 200 ms |
| GET /courses | 100.0% | 100.0% | 0.0% | CPU-intensive logic; min response (245 ms) exceeds threshold — 100% failure both runs |
| POST /login | 6.0% | 9.0% | +3.0% | Session overhead grows slightly under 100 users; more login requests cross 200 ms threshold |
| GET /api/download/256 | 72.0% | 80.0% | +8.0% | Bandwidth contention worsens under doubled concurrency; more responses exceed 200 ms |
| Overall Total | 56.8% | 60.0% | +3.2% | Higher concurrency increases assertion failures across most endpoints; only / remains stable |

4.3  Analysis & Discussion

Impact on /courses (100% Failure):  The Course Catalogue endpoint triggered assertion failures for every single request in both profiles. This is expected behaviour: the endpoint is explicitly designed to perform CPU-intensive work with latency scaling under concurrency. The minimum observed response time rose from 213 ms (50 users) to 245 ms (100 users) — both exceeding the 200 ms threshold — confirming that a 100% assertion failure rate is unavoidable and correctly demonstrates the Duration Assertion functioning as intended.

Impact on /notices (82% → 88% Failure):  The Notice Board injects a server-side random delay of 100–400 ms. With a 200 ms assertion threshold, all requests receiving a delay > 200 ms fail. The increase from 82% to 88% under 100 users demonstrates that higher concurrency amplifies queuing effects — more requests are effectively delayed beyond the threshold due to server-side resource contention at elevated thread counts.

Impact on /api/download/256 (72% → 80% Failure):  The bandwidth endpoint error rate worsened from 72% to 80% under doubled concurrency. With 100 threads simultaneously requesting large data payloads, bandwidth contention increases measurably — pushing more responses above the 200 ms assertion threshold. The maximum response time nearly doubled from 2237 ms to 3938 ms.

Impact on POST /login (6% → 9% Failure):  While still the best-performing endpoint, the Login service showed a slight increase in assertion failures from 6% to 9% under 100 users. Session creation overhead and connection pool pressure under doubled concurrency caused more login responses to exceed the 200 ms threshold.

Overall Error Rate (56.8% → 60.0%):  The total assertion failure rate increased from 56.8% to 60.0% when moving from 50 to 100 concurrent users. This is the expected behaviour: higher concurrency stresses the server's response time, causing more requests to breach the 200 ms Duration Assertion threshold. The Duration Assertion successfully generated a controlled mix of pass/fail events across all endpoints in both profiles, fulfilling the assignment's requirement.

5.  Conclusion

This performance evaluation demonstrates that the LoadLab application responds predictably to load increases from 50 to 100 concurrent users. The key findings are:

• The static Home Page and Login endpoints are the most resilient — maintaining relatively low response times — but both showed modest increases in assertion failures under 100 users, confirming that no endpoint is fully immune to concurrency pressure.

• CPU-intensive endpoints (Course Catalogue) consistently breach the 200 ms assertion threshold in both profiles, with average latency increasing from 443.9 ms to 519.2 ms under 100 users. This highlights a fundamental bottleneck requiring server-side optimisation (e.g., caching, horizontal scaling) to resolve.

• The Duration Assertion at 200 ms served its purpose as a diagnostic tool — triggering controlled failures that expose latency issues. The overall error rate increased from 56.8% to 60.0% as concurrency doubled, confirming the assertion correctly captures performance degradation under load.

• Throughput and bandwidth metrics nearly doubled from 50 to 100 users (2.50 → 4.94 req/s total; 155.4 → 307.1 KB/s received), demonstrating near-linear scaling behaviour and indicating the server has not yet reached saturation at 100 concurrent users.

Overall, this assignment provided valuable hands-on experience in configuring JMeter test plans, applying assertions, and interpreting performance data to draw meaningful engineering conclusions about scalability and bottleneck identification.